/**

\page dusime DUSIME key concepts

A good simulator, aside from generating the behaviour of the "real

thing" with the appropriate fidelity, will provide a number of extra

possibilities to its users (experimentators or trainees). An aircraft

simulation can normally be stopped and started in mid-air, and it is

often useful to have "playback" and "repeat" functions, with which one

can review a previous simulation, or jump back to a point where things

got interesting to have another try at the task.

On a distributed real-time system, such meta-functions have to be

implemented carefully. DUECA by itself gives the necessary tools to

create a distributed calculation process, but has no additional

facilities specifically for simulations. DUSIME is a (very thin and

small) application on top of DUECA that does all these things. A part

of DUSIME sends out DUECA events to make the rest of the simulation

stop, start, take a snapshot, test the hardware, etc.

Another part of DUSIME must be present in each of the modules that

participate in a DUSIME simulation, because each module must

understand the instructions to start, stop, etc. To make things easy

for developers, this part is all encapsulated into the code of the

HardwareModule and SimulationModule classes. By deriving their module

from a SimulationModule or HardwareModule, developers are ensured of

DUSIME compatibility.

\section dusime_trim Trim condition calculation

@warning Trim calculation is presently not implemented

A trim condition of a model is a model state, combined with values for

the input signals, that satisfies certain criteria. Take for example

trimming an aircraft model for horizontal flight. You want to find a

power setting, control input and aircraft attitude so that after the

start of the simulation, and with identical input signals:

<ul>

<li> The speed remains constant

<li> The airplane flies level (i.e., vertical speed is 0)

<li> The flight is symmetric

<li> The airplane keeps its pitch attitude

</ul>

Finding such a trim condition is done by an iterative process. In this

process, state variables, derivatives of state variables and input

variables can have different roles:

<ol>

<li> A \em Constraint. This is a part of the vehicle state

that may be set to a specific value by the experiment leader. For

example the aircraft position is a constraint.

<li> A \em Target. This is a part of the state -- or of the

derivative of the state, that has to be within certain bounds. For

example, you want the pitch acceleration ( \f$ dq/dt \f$ ) to  be zero, so

that the aircraft does not start pitching when the simulation is

started.

<li> A \em Control. This is something the initial condition

calculation can use to achieve the targets. For example the pitch

attitude may be varied until the flight path is level, and the

elevator input is varied until the pitch acceleration is zero.

</ol>

It is possible to specify the constraints, targets and controls for a

number of different initial condition modes, currently, DUSIME knows

the following modes:

<ol>

<li> FlightPath, with specification of the flight path angle and

speed, gets a power setting, attitude and elevator/trim input.

<li> Speed, with specification of the speed and power setting, gets a

flight path angle, attitude and elevator/trim input.

<li> Ground, for a calculation of an initial condition on the ground,

with the gear down and at zero speed

</ol>

It is important to realise that DUSIME knows actually nothing about

your model, and that it cannot do the trimming without the module's

help. Also, the initial condition modes are just conventions. In the

model you have to specify all variables (state, derivative of state or

input) that play a role in the inco calculation, and specify which

role they play for each of the inco modes. You also have to provide

the calculation for each of the inco modes.

\section dusime_snap Snapshot taking and loading

One of the luxuries of simulation is that you can repeat the mistakes

you have made in a simulation. A simulator can remember what has

"happened", and restore a state of the model and hardware that was

recorded in the past. This is popularly called a "snapshot". A

simulation snapshot is a complete (!) copy of the simulation model

state. It is possible to reload this copy, and so jump back in the

simulation to a previous point.

The state that you calculate in a trim calculation for the model does

not differ from the state you take in a snapshot. For economy of

programming (not computer cycle use), DUSIME combines these two

mechanisms. If you calculate a trim condition for your model, a

snapshot is made of that trimmed state, and this snapshot is used to

transfer the state of the trim model to the state of the model used in

the calculations.

\section dusime_state States of the simulation

A very important thing to remember in DUECA/DUSIME, is that there are

two different levels of "state" for a module. The first level of state

concerns the "DUECA" aspect of a module. Regarding DUECA, a normal

module can be in one of the following states:

<ol>

<li> Off. The module is not running, i.e. its main activity (normally its

work in the simulation) is not switched on.

<li> Working. The module is working, i.e. its main activity is switched

on. Normally this means that triggering events, such as the arrival of

data on a channel, give rise to scheduling of the activity, and the

activity is subsequently invoked.

</ol>

An additional state is possible for a module with a CriticalActivity,

in general these kinds of activity are only used by HardwareModule

derived modules:

<ol>

<li> Safe. The module is running in safe mode. This means that the

CriticalActivity of the module is calling an alternate callback method

of the module. This method must drive the hardware to a safe state and

keep it there.

</ol>

Now DUECA does not directly tell the Activity objects when to start

and when to stop, because it does not know exactly which activities

should be started and stopped. Instead, one of four virtual methods

from the Module base class are called:

<ol>

<li> Module::initialStartModule. This method is only useful for

modules with CriticalActivity objects, and these objects should be

switched on at this point (using Activity::switchOn). The effect will

be that the activity will start running in safe mode.

<li> Module::startModule. Normal modules should switch on their

activities when this method is called (using Activity::switchOn, with

the provided TimeSpec). Modules with CriticalActivity objects switch

their activity to the working mode, with the

CriticalActivity::switchWork method.

<li> Module::stopModule. Normal modules can switch their activities

off in this method (Activity::switchOff), Modules with

CriticalActivity objects switch their activity to the safe mode with

the CriticalActivity::switchSafe method.

<li> Module::finalStopModule. Again, this method is only useful for

modules with CriticalActivity objects, and these objects should be

switched off at this point (using Activity::switchOff). From the point

defined by the provided TimeSpec on, the activity will no longer be

running.

</ol>

The additional advantage of this mechanism is that your module at

least knows that a mode change takes place. The effect is that, seen

from a DUECA viewpoint, there are three possibilities; a module is

switched off, and its activities will not be invoked, a module is

running in safe mode, or a module is working.

Hardware devices controlled from a DUECA module will always be

controlled from safe mode initially, then, if the module has indicated

it is ready to go into working mode, the control may be from working

mode, and before dueca is exited -- barring program crashes of course

-- control will again be from safe mode. Ergo, hardware devices must

be able to switch "elegantly" between safe and working mode, and they

must be able to start and end participation in a DUECA-controlled

simulation in safe mode.

Simulation functionality is added by DUSIME, with a number of states

that are only handled within DUECA's working mode, and so these states

must all fit within the method that is called by the module's main

Activity. A module should call the Module::getAndCheckState method to

find out which DUSIME state is active. A number of states are defined,

and of these the following are "steady" states:

<ol>

<li> SimulationState::Inactive. This is the initial DUSIME mode that a

HardwareModule will enter after its CriticalActivity has been switched

from Safe to Working. The controlled hardware device must be able to

make a smooth transition from the commands give to it in Safe mode to

the commands given to it in (Working + Inactive) mode, and vice versa.

<li> SimulationState::HoldCurrent. This is a state in which a

HardwareModule or a SimulationModule is prepared to participate in the

simulation. In HoldCurrent, hardware is kept in a state from which at

any moment the simulation may start, and the model is kept "frozen",

i.e. there is no update with time.

<li> SimulationState::Advance. In this state, the simulation is

running normally. The model state is advanced to keep in pace with the

progress of wall clock time, hardware inputs are read out to provide

the control to the model and output hardware provides cueing on the

basis of the model output.

<li> SimulationState::Replay. This state is like Advance, but the

input to the simulation is from a previously recorded inputs, instead

of from the hardware and the pilot's control actions.

</ol>

Since some hardware cannot switch state infinitely fast, a number of

transition states have been defined. A module stays in a transition

state until it indicates that the transition is complete, then it will

be in the final state of the transition. Note that for different

modules the transition to the final state may be asynchronous; the

control loading system may have reached HoldCurrent while the motion

system is still in transition.

<ol>

<li> SimulationState::Inactive_HoldCurrent. The transition from

Inactive to HoldCurrent. For example for the SRS motion system this

means moving the motion system from the down position to the neutral

or center position for the simulation.

<li> SimulationState::Calibrate_HoldCurrent. This transition state is

always entered from HoldCurrent and returns to HoldCurrent. In this

state the hardware may perform calibration, if needed. If no

calibration is needed, immediately indicate that the transition is

complete, with HardwareModule::transitionComplete(), and do the action for

HoldCurrent.

<li> SimulationState::Advance_HoldCurrent. This transition state is

entered from the Advance state. The hardware must return to the

starting position/state for HoldCurrent.

<li> SimulationState::HoldCurrent_Inactive. This transition state is

entered from the HoldCurrent state. The hardware must return to the

Inactive state, e.g., the motion system goes down.

</ol>

Modules derived from the HardwareModule class may receive any of the

above states after calling the HardwareModule::getAndCheckState

method. Modules that "only" do model calculations do not need this

full set of states, therefore the SimulationModules::getAndCheckState

method for these modules only returns three different states:

HoldCurrent, Replay and Advance. All other states, the Inactive state

and all transition states, are mapped onto the HoldCurrent state.

Hardware that is controlled by a DUSIME module, may expect the

following actions from that module:

<table>

<tr>

<td> DUECA activities Off </td>

<td> No communications </td>

</tr>

<tr> <td> DUECA activities Safe </td> <td> The hardware must be driven

to a safe state, and it must be kept there. This mode is the initial

mode in which communication with DUECA is established, and it is

normally also the mode in which communication will be stopped. In a

normal, controlled shutdown, this mode is entered from the Inactive

mode. This mode may also be entered at any time due to an error

condition in DUECA. In this latter case the transition to the safe

position need not be "elegant".  </td> </tr>

<tr>

<td> DUSIME Inactive </td>

<td> The hardware is kept in a down / inactive / inert

state. Transitions to and from Inactive and Safe may occur

instantaneously, and should be handled gracefully (i.e. no bumps,

awkward movements) </td>

</tr>

<tr>

<td> DUSIME Inactive_Holdcurrent </td>

<td> The hardware is brought to the starting position for cueing. </td>

</tr>

<tr>

<td> DUSIME HoldCurrent </td>

<td> The hardware is kept in the starting position for cueing. </td>

</tr>

<tr>

<td> DUSIME Calibrate_Holdcurrent </td>

<td> From the starting position for cueing, perform a calibration, if

necessary, and return to that starting position. </td>

</tr>

<tr>

<td> DUSIME Active</td>

<td> Provide cues/obtain inputs. </td>

</tr>

<tr>

<td> DUSIME Replay</td>

<td> For hardware that only provides cues, this mode might be the same

as the Active mode. Hardware that provides input may somehow replay

the input (e.g. the control loading simulation replays the movements

by adding the recorded pilot force trace to the input).</td>

</tr>

<tr>

<td> DUSIME Active_Holdcurrent</td>

<td> Return to starting position for cueing </td>

</tr>

<tr>

<td> DUSIME HoldCurrent_Inactive</td>

<td> go to the "down" position </td>

</tr>

</table>

Note that not all hardware needs to make distinctions between all these

modes, the control strategy applied may map onto a limited number of

modes for the hardware. This mapping may be easily implemented within

the DUSIME HardwareModule controlling the hardware.

\section dusime_ready Indicating that you are ready to leave a mode

DUECA is a distributed system, and the state machines that control

DUECA and DUSIME must take this into account. For a DUSIME hardware

module, things are complex, but the rules are simple; if the module

enters a transition state, like Inactive_HoldCurrent, it can say in

that state as long as it needs, and only when it indicates that it is

ready, with transitionComplete(), the DUSIME controller can proceed to

the next state.

Simulation modules only get the "final" states, and implicitly

indicate to DUSIME that their transitions are complete. However, in

some cases it might be better to not do this. Typical examples are

modules that handle a configuration window for an experiment, and a

transition from HoldCurrent to Advance should only take place after

the experiment is properly configured. In that case, use the complete

list of parameters to the dueca::SimulationModule::getAndCheckState()

function, and only set the second parameter to true when the module is

ready to continue to other states.

*/