/** \page app_develop Application Development and Deployment Design

\section app_purpose Purpose of this document

In the "early years" of dueca, and especially with the first

experiments on the SIMONA Research Simulator (SRS), it became apparent

that support in version control and deployment of an application over

several machines, and in different environments, is an essential part

of the development environment of DUECA.

This document outlines how the use of a version control system (CVS),

a special purpose script (dueca-work) and adherence to a specific

directory structure for development of application could assist in the

re-use of existing code and modules, and in the quick deployment and

continued development on multi-node environments such as the

Human-Machine Interaction laboratory (HMI lab) and the SRS. The

specific goals for this design are:

<ul>

<li> Facilitate development of modules and applications.

<li> Promote re-use of developed modules

<ul>

<li> Unmodified, through the easy and safe inclusion of generically

applicable (hardware related or otherwise) modules.

<li> Instantiated, through a mechanism for including a template module

and the ability to instantiate such a module for one's specific

purposes.

<li> Branched, through copying and subsequent modification of an

existing module.

</ul>

<li> Facilitate deployment of an application on multi-node dueca

environments.

<li> Facilitate running on a multi-node dueca environment.

</ul>

Because of the mixed nature of the design, no formal description

method is used, rather, this description is in text/narrative format.

\section app_mech Applied mechanisms and tools

To enable development on different platforms, CVS will be used as a

versioning and repository tool. Hovever, CVS typically handles

complete projects, and a dueca application should be able to use,

without copying and inclusion, generic parts. Therefore applications

can be based on the check-out and development of several CVS projects,

and a script will be used to facilitate this multiple use of CVS.

The top-level objects that are checked into the CVS versioning system

are then the application itself, including its "own" modules,

configuration scripts and its proper run environment, a "generic"

development tree, that contains generic modules (e.g. for a joystick)

and template instantiatable modules, and possibly site-specific

generic trees.

The script (or scripts) should be able to

<ul>

<li> Support compilation of deployment. The script exports the

application, and based on hostname or an explicitly supplied "build

name", extracts the necessary components from the application tree

itself and the generic or site-specific trees

<li> Support development on a deployment platform. Works as above,

but also checks out the sources.

<li> Support initial creation and set up of a new application.

<li> Support the inclusion of test results, including marking

versions and documentation of the run environment.

<li> Support application development

<ul>

<li> Addition of new modules

<li> Addition of source files to a module

<li> addition of necessary run data

<li> Inclusion/use of generic modules

<li> Addition of communication objects

</ul>

</ul>

The script and/or the cvs environment should provide some constraints,

for example developers should not have read/modification access to all

modules (possible through the use of commitinfo script). Remote use of

a cvs repository should also be possible.

The communication objects for a deployment should be tailored to the

modules used.

\section app_dirstruc Directory structure design

The need for "borrowing" generic modules from other cvs trees dictates

to some extent the design of the directory structure. Using MyApp as

an example application name, the directory structure is given as:

\code

MyApp                Main directory

MyApp/MyApp        The directory with application-specific run files

and modules

MyApp/SomeGenericModule

A directory with the Makefile/directories that

pertain to a generic, re-used module

MyApp/SomeGenericModule/SomeGenericModule

The directory with the source code for the

SomeGenericModule

MyApp/SomeGenericModule/comm-objects

The directory with communication objects needed by

SomeGenericModule

MyApp/MyApp/modules.solo

A file that lists all modules needed (not including

directories with communication objects) needed by an

application, in the form:

MyApp/MyModule

SomeGenericModule/SomeGenericModule

The extension, .solo, indicates that this list of

modules is needed either on a machine named solo

(which is seldom the case), or for a run-name "solo"

A file modules.dutmms4 would list the modules to be

used on the machine dutmms4

MyApp/MyApp/MyModule

Directory for a module specifically developed or

adapted for this application. This directory contains

source files and a make file for this module. This

make file may be a symbolic link to a generic make

file.

MyApp/MyApp/MyModule/comm-objects.lst

List with communication objects needed for this

module. In the form of:

MyApp/comm-objects/MyCommObject.dco

SomeGenericModule/comm-objects/SomeCommObject.dco

MyApp/MyApp/comm-objects

Directory for communication objects specifically

developed or adapted for this application. When using

the code generator, each object is created from a

.dco file (dueca communication object)

MyApp/MyApp/run

Run directory for the application. Sub directories of

this directory define the dueca environment + machine,

so:

MyApp/MyApp/run/solo/solo

Run directory for running development on a single

machine

MyApp/MyApp/run/mmslab/dutmms1

Run directory for the files used by the dutmms1 on the

HMI laboratory

MyApp/MyApp/run/datafiles

ALL data files needed for running an

application. Bitmaps, experiment conditions, etcetera.

MyApp/MyApp/run/solo/solo/links.lst

List with data files you would liked to be linked to

this directory. In sh(1) globbing style, so a line

with "*.dat" would link all .dat files in the

datafiles directory to this directory. This is optional.

datafiles directory to the present one

MyApp/MyApp/run/solo/solo/clean.lst

List with types of files you would like cleaned from

the present run directory, when mrproper (or

realclean) is called. This is optional.

\endcode

There should always be a modules.solo file, which list the modules

used when developing (or running) on a single machine.

A default makefile for modules will be supplied with the dueca

installation. If only c++ and c files are needed, and no additional

include flags are required, running this makefile will usually

correctly build a module. To use this makefile, it will be copied by

the script when making a new module.

The makefile may include (and if it works

it really should include) a generic makefile "tail" supplied in the

dueca distribution. Only if you have special requirements on the

sources, you need to edit this makefile.

For special work, where building on different machines may differ,

a Makefile with the build name/machine name extension may be used. A

Makefile.dutmms4 in the cvs sources for example would be checked out,

and a symbolic link to this file will be made. In the current version,

this is not yet implemented.

This applies both to the application directory and the module

directories. The make command should be given from the application

directory (MyApp/MyApp).

\section app_module Module directory

An application normally consists of one to several modules. Modules

can always be compiled independently of each other, they should only

depend on the dueca installation, communicated objects (.dco files and

their generated .hxx/.cxx offspring) and possibly on external

libraries.

To get a fitting compilation of the modules and the objects that they

communicate with, each module should specify a list of the

communication objects that it uses, in the file

"comm-objects.lst". This file is included in the cvs system, and it

can be used to determine which communication objects need to be

checked out for a specific application. The Makefile also creates a

"comm-objects.h" file for easy inclusion (and exclusion of errors)

from this file.

If the version system contains a Makefile.machine, where machine is

the build name or machine name for the module directory, then this

Makefile is checked out and used. Otherwise, if it contains a

Makefile, this makefile is checked out, and if it does not contain a

Makefile, a link to the dueca-supplied Makefile for modules is used.

\section app_rundir Run directory

One of the annoying problems in deploying an application on a certain

machine is keeping up with all the data files needed for running the

application. Different modules often read different data files, which

in general mist be present in the directory where dueca is

started. Files generated by the application are also left there, and

when it is time to clean-up the run directory, one has to find out

which of the many files must be cleaned out, and which contain

valuable (configuration) data. To solve this problem, the following

solution is proposed:

<ul>

<li> A directory is created which contains ALL the data files (in

itself or in sub-directories) for ALL dueca environments and

machines.

<li> Each environment (a lab, a simulator, a single development

machine) has a sub-directory, within which each machine in an

environment has it own sub-directory with machine specific

configuration files.

<li> For development, a shared machine/build name, namely "solo" is

used, So there is always an "environment" directory solo, with a run

directory "solo/solo" which contains the single-machine development

and test environment.

<li> A run directory has the following files committed in cvs:

<ol>

<li> dueca.cnf: The configuration file for start-up of dueca

<li> dueca.mod (optionally): For node 0 only, the file defining

the application

<li> links.script: A script file that makes links to the files or

directories under "datafiles" that are needed for

running the application on that specific machine.

<li> clean.script: A script file that cleans, all

files to be removed at "really really cleaning" time. I suggest

this includes the generated data. At that time all links from

links.lst are also removed.

</ol>

</ul>

\section app_adhere Enforcing the development model

It is difficult to enforce strict adherence to the directory structure

and proposed rules given above. I hope that developers will see the

advantages of sticking to a set of common rules for getting more

uniform applications. Certainly I (and I assume that later also other

dueca "cracks") will no longer assist in porting applications that

have been hacked together, not using CVS nor the supplied scripts, It

is also likely that as the script(s) evolve, more and more checking

will be done, and ease of use will go up, meaning that there is no

longer an argument to not use these.

*/