/** \page example Simulation example

This section describes a large part of the SpacePlane model simulation

that has been used as an example in this manual.

This simulation derived from an experiment implemented on the

Human-Machine Systems lab of the

\htmlonly <A href="http://www.cs.lr.tudelft.nl/"> Control and Simulation </A> \endhtmlonly

\latexonly Control and Simulation \endlatexonly

Division, Faculty of Aerospace Engineering

The simulation model of the equations of motion of the spaceplane,

including the control augmentation system, was programmed in one

Simulink model. Using the Real-Time Workshop (RTW) toolbox, C code was

generated for this Simulink model. Input of the RTW code are the pitch

and roll stick inputs, and the output of the code is an output vector

with all spaceplane motions.

The simulation was controlled with a control loaded side stick. This

stick uses a hydraulic servo motor to simulate mechanical properties,

in this case the mechanical properties of a passive (mass, spring and

damper) stick.

Instrument display was done on a PC with a fast graphics card. The

display is drawn using the output of the RTW model.

In addition to the display output, output to file was needed. The

stick force and position data, and the spaceplane model output data,

were recorded at a 50 Hz rate. Output to the control position of the

experiment leader was also needed, during the run some key variables

were shown in a window at the experiment leader's position, and key

variables at certain points in the experiment (points along the

pre-defined trajectory) were also shown and recorded. In this way it

was for example possible to see the sink rate at touch down directly

after a run.

\section example_flow

Data Flow diagram

\section example_bloks Modules (or Blocks)

The first step in designing the simulation is setting up a (DUECA)

data flow diagram. This diagram defines the modules used in the

simulation, and the data (types) they send to each other.

\image html spaceplane.png "DUECA data flow diagram for the spaceplane"

The data flow diagram contains the following blocks:

<ul>

<li> A module MMSLabStick, which reads the input force on the stick

(i.e. directly from hardware), calculates the dynamics of the stick

simulation, outputs the result of this calculation to the stick (to

hardware), and also sends out stick position and force over a channel.

<li> A module SpacePlane, which reads the stick input (position),

feeds that to the RTW model, makes an update of the model, and sends

out the output vector of the model over a channel.

<li> A module DisplaySpace, which reads the spaceplane output vector,

and draws the instrument display.

<li> A module NumericOutput, which reads the spaceplane output vector

\em the stick position and force data, and prints these to a file.

<li> A module Evaluator, which reads the spaceplane output vector,

calculates the deviation from the nominal path and presents this and a

few other key variables in a window.

</ul>

These are all the modules that are needed from a model

point-of-view. One other module is added for practical reasons. As

will be explained in the following, the stick control loading and the

spaceplane model run on different computers. The control loading has

to run at a high update rate (2000 Hz), and it also produces data at

2000 Hz. The spaceplane model runs slower (50 Hz), and thus it needs

every 40th output of the control loading model. Now, without

modifications, DUECA would send the stick data at 2000 Hz to the other

computers, where only 1/40 th of this data would be used. An

additional routine, called RateReduction, which runs in the computer

with the stick module, reads the stick data and writes with only 50

Hz.

\section example_communicate Data types communicated over channels

The following model-related data types were communicated over the channels:

<ul>

<li> PrimaryControls, a class that contains all data from the side

stick.

<li> SpacePlaneY, the output vector of the space plane.

<li> DisplaySelect, the selected display.

</ul>

Here is the file input for the code generator:

\verbatim

(Type double)

(EventAndStream PrimaryControls

(double stick_roll )

(double stick_pitch )

(double roll_moment)

(double pitch_moment)

(double roll_rate)

(double pitch_rate))

(EventAndStream SpacePlaneY

(double Y 29 ))

(Type int )

(Event DisplaySelect

(int type ))

\endverbatim

\section example_code Header and implementation files

Here are all header and implementation files for the modules.

<ul>

<li> First the stick input:

\include MMSLabStick.hxx

\include MMSLabStick.cxx

These files use an auxiliary class PassiveStick, which is not

included in this manual. Since the HardwareModule class is not

complete yet, this uses the SimulationModule class.

<li> The rate reduction class, which simply reads in the stick input

and writes it back again. By specifying an update rate for its

activity, it assures itself of reading only at the proper times,

and writing with the proper TimeSpec.

\include RateReduction.hxx

\include RateReduction.cxx

<li> The space plane module.

\include SpacePlane.hxx

\include SpacePlane.cxx

This module's calculations are based on a model that was

implemented in SimuLink and converted with Real-Time

Workshop. For the conversion you should specify the "real time

malloc" option, this will enable the code to start up several

instances of the same model in one program. A script called

"correctSimulink", which is installed with the DUECA program,

makes some small corrections to the generated Simulink code. Run

this once on the RTW files.

<li> The numeric output module.

\include NumericPlaneOutput.hxx

\include NumericPlaneOutput.cxx

<li> The evaluator shows a window on the screen of the experimenter's

PC, with current values and overview values. The windowing

toolkit that is used is gtk+. With a GUI designer glade, it is

easy to draw such a window. A few callback functions (which must

have a "C" linkage specification) are written to handle the

events from the interface.

\include Evaluator.hxx

\include Evaluator.cxx

</ul>

You see that there is certainly some code writing in making a

module. Fortunately, most of the code can be generated by the script

"newModule". This script is installed with the dueca program. Run it

with the name of your new module (e.g. MyModule), and it will produce

a MyModule.cxx and a MyModule.hxx file. The comments in these files

should guide you with the job of tailoring the module to your

application.

\section example_model Model configuration file.

The example simulation runs on 3 PC's, connected with fast

ethernet. The division of modules over the PCs is as follows:

<ul>

<li> Node number 0 is the experimenter's PC. DUECA runs all

administrative duties, and also reads the model data from the

dueca.mod file, on node number 0. The following modules are

implemented here:

<ul>

<li> The space plane module

<li> The numeric output module

<li> The evaluator module.

</ul>

<li> Node number 1 is the control loading PC, running QNX. This PC

does the control loading simulation, and it runs the rate reduction

module. Since this is the fastest-running PC (cycle-time wise), it is

the clock master.

<li> Node number 2 is the display PC. It only runs the display

module.

</ul>

Scheme is a full-fledged programming language, and therefore the model

configuration file in fact contains a simple program. To make life a

bit easier, we can define often-used variables in the model

configuration file as Scheme variables. That is what this first part

of the configuration file does:

\dontinclude dueca.mod

\skip document

\until feedback-timing

Dueca itself also needs some modules. Here they are:

\skip itself

\until )))

And then the modules for the spaceplane are created.

\skip spaceplane

\until complete

Notice the use of the "if (equal? this-node-id cl-node)" construction. All

modules defined within the if statement are designated for that

node. If this is not used, the modules will be created in all nodes in

parallel, and this would certainly give channel conflicts.

*/

<code>

tud0034882:dapps repa$ dueca-project new SpacePlane

</code>

<code>

tud0034882:SpacePlane repa$ dueca-project borrow-module Generic/multi-stick

</code>

<code>

tud0034882:SpacePlane repa$ dueca-project new-module space-plane

</code>

<code>

tud0034882:space-plane repa$ new-module rtw88 SpacePlane

Give the name of the RTW model (must be different!):  : spaceplane

A description for the activity:  : update dynamics

Generate xml parsers (and integrate into module)? [Y/n]:  : n

</code>

<code>

tud0034882:space-plane repa$ dueca-project add-source SpacePlane.?xx

</code>